# ELEC 204 / FALL 18

# LAB PROJECT

WASHING MACHINE

## Hasan Can Aslan

Description

The washing machine has 4 states of operation: WASH, SPIN, RINSE, IDLE. And has 5 states of mode: SHEET, WHITE, COTTON, COLOR, RAPID. The washing machine synchronous circuit with clock signal CLK. There are two external 1-bit inputs: START and RESET buttons. The external output is a 2-bit signal S1S0 that shows which state the washing machine is in (WASH, SPIN, RINSE, IDLE are 00, 01, 10, 11, respectively). WASH, SPIN and RINSE states use a down counter. The counters all use a 1-bit internal variable DEC. The counters synchronously decrement once each second for DEC = 1, It has a single internal output, ZERO, which is 1 whenever the counter reaches zero and is 0 otherwise.
The counters for WASH, SPIN and RINSE states count for different integers for every states of mode. Next, the counter for WASH state starts counting down (S1S0 = 00). As soon as this counter reaches zero, ZERO becomes 1, the counter for WASH state resets and the system transitions to SPIN state. Then, the counter for SPIN state (S1S0 = 01) starts counting down. As soon as this counter reaches zero, ZERO becomes 1 again, the counter for SPIN state resets and the system transitions to RINSE state. In this final operational state which is RINSE (S1S0 = 10), the counter for RINSE state starts counting down. As soon as this counter reaches zero, ZERO becomes 1 again, the counter for SPIN state resets and the system transitions to RINSE state. RESET input aborts the entire washing procedure, sets DEC = 0, and the machine resets to the IDLE state (S1S0 = 11).

# Codes

# Clock

library IEEE;

use IEEE.STD_LOGIC_1164.ALL;

USE IEEE.STD_LOGIC_ARITH.ALL;

USE IEEE.STD_LOGIC_UNSIGNED.ALL;

entity clock_divider is

Port (

MCLK : in STD_LOGIC;

HUNDREDHZCLOCK : out STD_LOGIC);

end clock_divider;

architecture Behavioral of clock_divider is

SIGNAL COUNTER : STD_LOGIC_VECTOR(18 DOWNTO 0) := "0000000000000000000";

begin

CLK_PROCESS: PROCESS(MCLK)

BEGIN

if (MCLK'EVENT AND MCLK = '1') then

if COUNTER <= "0011110100001001000" then

COUNTER <= COUNTER + 1;

else

COUNTER <= "0000000000000000000";

end if;

end if;

END PROCESS;

HUNDREDHZCLOCK <= '1' WHEN COUNTER = "0001111010000100100" else '0';

end Behavioral;

# Decoder

LIBRARY IEEE;

USE IEEE.STD_LOGIC_1164.ALL;

ENTITY SevSeg_Decoder IS

PORT ( INPUT : IN STD_LOGIC_VECTOR (4 DOWNTO 0);

SEVSEG_BUS : OUT STD_LOGIC_VECTOR (6 DOWNTO 0));

END SevSeg_Decoder;

ARCHITECTURE BEHAVIORAL OF SevSeg_Decoder IS

BEGIN

WITH INPUT SELECT SEVSEG_BUS <=

"0000001" WHEN "00000", --0

"1001111" WHEN "00001", --1 or I

"0010010" WHEN "00010", --2

"0000110" WHEN "00011", --3

"1001100" WHEN "00100", --4

"0100100" WHEN "00101", --5

"0100000" WHEN "00110", --6

"0001111" WHEN "00111", --7

"0000000" WHEN "01000", --8

"0000100" WHEN "01001", --9

"0001000" WHEN "01010", --A

"1100000" WHEN "01011", --B

"0110001" WHEN "01100", --C

"1000010" WHEN "01101", --D

"0110000" WHEN "01110", --E

"1110001" WHEN "01111", --L

"1100001" WHEN "10000", -- W Part 1

"1000011" WHEN "10001", -- W Part 2

"0001001" WHEN "10010", -- N

"0011000" WHEN "10011", -- P

"0111001" WHEN "10100", -- R

"0100100" WHEN "10101", -- S

"1110000" WHEN "10110", -- T

"1001000" WHEN "10111", -- H

"0000000" WHEN "11000", -- K

"0011000" WHEN "11001", -- P

"1111111" WHEN "11010", -- Blank

"0100100" WHEN "11011", --

"1110110" WHEN "11100", --

"0100001" WHEN "11101", --

"0110000" WHEN "11110", --

"1110001" WHEN "11111", --

"1111111" WHEN OTHERS;

END BEHAVIORAL;

# Main

library IEEE;

use IEEE.STD_LOGIC_1164.ALL;

entity main is

Port (

Clock : in std_logic;

start : in std_logic;

reset : in std_logic;

leds 	: out std_logic_vector(9 downto 0);

userMode : in std_logic_vector(4 downto 0);

SevSegData : out std_logic_vector(6 downto 0);

SevSegDriver : out std_logic_vector(7 downto 0));

end main;

architecture Behavioral of main is

signal ledOut : std_logic_vector(9 downto 0);

signal time_sn : std_logic_vector(4 downto 0);

signal dataSevSeg : std_logic_vector(4 downto 0);

signal hz : std_logic;

signal tempLed : std_logic_vector (9 downto 0);

Component washing_machine

Port (

CK : in std_logic;

start : in std_logic;

reset : in std_logic;

userMode : in std_logic_vector(4 downto 0);

state : out std_logic_vector(9 downto 0);

time_sn  : out std_logic_vector(4 downto 0));

end Component;

Component clock_divider

PORT(

MCLK : IN std_logic;

HUNDREDHZCLOCK : OUT std_logic

);

END Component;

COMPONENT SevSeg_Decoder

PORT(

INPUT : IN std_logic_vector(4 downto 0);

SEVSEG_BUS : OUT std_logic_vector(6 downto 0)

);

END COMPONENT;

COMPONENT SevSeg_Driver

PORT(

time_sn : IN std_logic_vector(4 downto 0);

leds : IN std_logic_vector(9 downto 0);

userMode : IN STD_LOGIC_VECTOR(4 downto 0);

CLK : IN std_logic;

start : IN STD_LOGIC;

SEV_SEG_DATA : OUT std_logic_vector(4 downto 0);

SEV_SEG_DRIVER : OUT std_logic_vector(7 downto 0)

);

END COMPONENT;

begin

clk : clock_divider Port Map(Clock, hz);

wsh : washing_machine Port Map(hz, start, reset, userMode, templed, time_sn);

driver : SevSeg_Driver PORT MAP(time_sn, tempLed, userMode, hz, start, dataSevSeg, SevsegDriver);

decoder : SevSeg_Decoder PORT MAP(dataSevSeg, SevSegData);

leds <= tempLed;

end Behavioral;

# Driver

library IEEE;

USE IEEE.STD_LOGIC_1164.ALL;

USE IEEE.STD_LOGIC_ARITH.ALL;

USE IEEE.STD_LOGIC_UNSIGNED.ALL;

USE IEEE.NUMERIC_STD.ALL;

ENTITY SevSeg_Driver IS

PORT (

time_sn : IN STD_LOGIC_VECTOR (4 DOWNTO 0);

leds : IN STD_LOGIC_VECTOR(9 downto 0);

userMode : IN STD_LOGIC_VECTOR(4 downto 0);

CLK : IN STD_LOGIC;

start : IN STD_LOGIC;

SEV_SEG_DATA : OUT STD_LOGIC_VECTOR(4 DOWNTO 0);

SEV_SEG_DRIVER : OUT STD_LOGIC_VECTOR (7 DOWNTO 0));

END SevSeg_Driver;

ARCHITECTURE BEHAVIORAL OF SevSeg_Driver IS

-- Characters

signal A : std_logic_vector(4 downto 0);

signal B : std_logic_vector(4 downto 0);

signal C : std_logic_vector(4 downto 0);

signal D : std_logic_vector(4 downto 0);

signal E : std_logic_vector(4 downto 0);

signal L : std_logic_vector(4 downto 0);

signal W1 : std_logic_vector(4 downto 0);

signal W2 : std_logic_vector(4 downto 0);

signal N : std_logic_vector(4 downto 0);

signal P : std_logic_vector(4 downto 0);

signal R : std_logic_vector(4 downto 0);

signal S : std_logic_vector(4 downto 0);

signal T : std_logic_vector(4 downto 0);

signal H : std_logic_vector(4 downto 0);

signal K : std_logic_vector(4 downto 0);

signal O : std_logic_vector(4 downto 0);

signal Blank : std_logic_vector(4 downto 0);

SIGNAL COUNTER : STD_LOGIC_VECTOR(2 DOWNTO 0) := "000";

BEGIN

--INCREMENT COUNTER

A <= "01010"; --A

B <= "01011"; --B

C <= "01100"; --C

D <= "01101"; --D

E <= "01110"; --E

L <= "01111"; --L

W1 <= "10000"; -- W Part 1

W2 <= "10001"; -- W Part 2

N <= "10010"; -- N

P <= "10011"; -- P

R <= "10100"; -- R

S <= "10101"; -- S

T <= "10110"; -- T

H <= "10111"; -- H

K <= "11000"; -- K

Blank <= "11010"; -- Blank

PROCESS_CLK : PROCESS(CLK)

BEGIN

if rising_edge(CLK) then

if COUNTER = "111" then

COUNTER <= "000";

ELSE

COUNTER <= COUNTER + 1;

end if;

end if;

END PROCESS PROCESS_CLK;

SEV_SEG_DATA <=

-- Washing Modes

S		  WHEN COUNTER = "000" and userMode = "10000" and start = '0' ELSE

H	     WHEN COUNTER = "001" and userMode = "10000" and start = '0' ELSE

E		  WHEN COUNTER = "010" and userMode = "10000" and start = '0' ELSE

E		  WHEN COUNTER = "011" and userMode = "10000" and start = '0' ELSE

T  	  WHEN COUNTER = "100" and userMode = "10000" and start = '0' ELSE

Blank   WHEN COUNTER = "101" and userMode = "10000" and start = '0' ELSE

"00001" WHEN COUNTER = "110" and userMode = "10000" and start = '0' ELSE

"00001" WHEN COUNTER = "111" and userMode = "10000" and start = '0' ELSE

W1		  WHEN COUNTER = "000" and userMode = "01000" and start = '0' ELSE

W2	     WHEN COUNTER = "001" and userMode = "01000" and start = '0' ELSE

H		  WHEN COUNTER = "010" and userMode = "01000" and start = '0' ELSE

"00001" WHEN COUNTER = "011" and userMode = "01000" and start = '0' ELSE

T  	  WHEN COUNTER = "100" and userMode = "01000" and start = '0' ELSE

E 		  WHEN COUNTER = "101" and userMode = "01000" and start = '0' ELSE

"00001" WHEN COUNTER = "110" and userMode = "01000" and start = '0' ELSE

"00010" WHEN COUNTER = "111" and userMode = "01000" and start = '0' ELSE

C		  WHEN COUNTER = "000" and userMode = "00100" and start = '0' ELSE

"00000" WHEN COUNTER = "001" and userMode = "00100" and start = '0' ELSE

T		  WHEN COUNTER = "010" and userMode = "00100" and start = '0' ELSE

T		  WHEN COUNTER = "011" and userMode = "00100" and start = '0' ELSE

"00000" WHEN COUNTER = "100" and userMode = "00100" and start = '0' ELSE

N	     WHEN COUNTER = "101" and userMode = "00100" and start = '0' ELSE

"00001" WHEN COUNTER = "110" and userMode = "00100" and start = '0' ELSE

"00101" WHEN COUNTER = "111" and userMode = "00100" and start = '0' ELSE

C		  WHEN COUNTER = "000" and userMode = "00010" and start = '0' ELSE

"00000" WHEN COUNTER = "001" and userMode = "00010" and start = '0' ELSE

L		  WHEN COUNTER = "010" and userMode = "00010" and start = '0' ELSE

"00000" WHEN COUNTER = "011" and userMode = "00010" and start = '0' ELSE

R  	  WHEN COUNTER = "100" and userMode = "00010" and start = '0' ELSE

Blank   WHEN COUNTER = "101" and userMode = "00010" and start = '0' ELSE

"00001" WHEN COUNTER = "110" and userMode = "00010" and start = '0' ELSE

"00000" WHEN COUNTER = "111" and userMode = "00010" and start = '0' ELSE

R		  WHEN COUNTER = "000" and userMode = "00001" and start = '0' ELSE

A	     WHEN COUNTER = "001" and userMode = "00001" and start = '0' ELSE

P		  WHEN COUNTER = "010" and userMode = "00001" and start = '0' ELSE

"00001" WHEN COUNTER = "011" and userMode = "00001" and start = '0' ELSE

D  	  WHEN COUNTER = "100" and userMode = "00001" and start = '0' ELSE

Blank   WHEN COUNTER = "101" and userMode = "00001" and start = '0' ELSE

Blank   WHEN COUNTER = "110" and userMode = "00001" and start = '0' ELSE

"00110" WHEN COUNTER = "111" and userMode = "00001" and start = '0' ELSE

-- States of America -_-

D       WHEN COUNTER = "000" and leds = "1000000011" and start = '1' ELSE

"00000" WHEN COUNTER = "001" and leds = "1000000011" and start = '1' ELSE

N		  WHEN COUNTER = "010" and leds = "1000000011" and start = '1' ELSE

E		  WHEN COUNTER = "011" and leds = "1000000011" and start = '1' ELSE

D   	  WHEN COUNTER = "100" and leds = "1000000011" and start = '1' ELSE

"00000" WHEN COUNTER = "101" and leds = "1000000011" and start = '1' ELSE

N   	  WHEN COUNTER = "110" and leds = "1000000011" and start = '1' ELSE

E   	  WHEN COUNTER = "111" and leds = "1000000011" and start = '1' ELSE

W1      WHEN COUNTER = "000" and leds = "0100000000" and start = '1' ELSE

W2 	  WHEN COUNTER = "001" and leds = "0100000000" and start = '1' ELSE

A		  WHEN COUNTER = "010" and leds = "0100000000" and start = '1' ELSE

S		  WHEN COUNTER = "011" and leds = "0100000000" and start = '1' ELSE

H  	  WHEN COUNTER = "100" and leds = "0100000000" and start = '1' ELSE

Blank   WHEN COUNTER = "101" and leds = "0100000000" and start = '1' ELSE

Blank	  WHEN COUNTER = "110" and leds = "0100000000" and start = '1' ELSE

time_sn WHEN COUNTER = "111" and leds = "0100000000" and start = '1' ELSE

S       WHEN COUNTER = "000" and leds = "0010000001" and start = '1' ELSE

P   	  WHEN COUNTER = "001" and leds = "0010000001" and start = '1' ELSE

"00001" WHEN COUNTER = "010" and leds = "0010000001" and start = '1' ELSE

N		  WHEN COUNTER = "011" and leds = "0010000001" and start = '1' ELSE

Blank	  WHEN COUNTER = "100" and leds = "0010000001" and start = '1' ELSE

Blank	  WHEN COUNTER = "101" and leds = "0010000001" and start = '1' ELSE

Blank	  WHEN COUNTER = "110" and leds = "0010000001" and start = '1' ELSE

time_sn WHEN COUNTER = "111" and leds = "0010000001" and start = '1' ELSE

R       WHEN COUNTER = "000" and leds = "0001000010" and start = '1' ELSE

"00001" WHEN COUNTER = "001" and leds = "0001000010" and start = '1' ELSE

N		  WHEN COUNTER = "010" and leds = "0001000010" and start = '1' ELSE

S		  WHEN COUNTER = "011" and leds = "0001000010" and start = '1' ELSE

E		  WHEN COUNTER = "100" and leds = "0001000010" and start = '1' ELSE

Blank	  WHEN COUNTER = "101" and leds = "0001000010" and start = '1' ELSE

Blank	  WHEN COUNTER = "110" and leds = "0001000010" and start = '1' ELSE

time_sn WHEN COUNTER = "111" and leds = "0001000010" and start = '1' ELSE

Blank;

SEV_SEG_DRIVER <=

"01111111" WHEN COUNTER = "000" ELSE

"10111111" WHEN COUNTER = "001" ELSE

"11011111" WHEN COUNTER = "010" ELSE

"11101111" WHEN COUNTER = "011" ELSE

"11110111" WHEN COUNTER = "100" ELSE

"11111011" WHEN COUNTER = "101" ELSE

"11111101" WHEN COUNTER = "110" ELSE

"11111110" WHEN COUNTER = "111" ELSE

"11111111";

END BEHAVIORAL;

# Wash

library IEEE;

use IEEE.STD_LOGIC_1164.ALL;

-- Uncomment the following library declaration if using

-- arithmetic functions with Signed or Unsigned values

use IEEE.NUMERIC_STD.ALL;

use IEEE.STD_LOGIC_ARITH.ALL;

use IEEE.STD_LOGIC_UNSIGNED.ALL;

-- Uncomment the following library declaration if instantiating

-- any Xilinx primitives in this code.

--library UNISIM;

--use UNISIM.VComponents.all;

entity washing_machine is

Port(

CK 	: in std_logic;

start : in std_logic;

reset : in std_logic;

userMode : in std_logic_vector(4 downto 0);

state : out std_logic_vector(9 downto 0);

time_sn  : out std_logic_vector(4 downto 0));

end washing_machine;

architecture Behavioral of washing_machine is

signal hz : std_logic;

signal tState : std_logic_vector(1 downto 0);

signal washC : integer;

signal spinC : integer;

signal rinseC : integer;

signal modes : std_logic_vector(3 downto 0);

begin

with userMode select washC <=

5 when "10000",

6 when "01000",

7 when "00100",

5 when "00010",

3 when "00001",

0 when others;

with userMode select spinC <=

3 when "10000",

4 when "01000",

5 when "00100",

3 when "00010",

2 when "00001",

0 when others;

with userMode select rinseC <=

3 when "10000",

2 when "01000",

3 when "00100",

2 when "00010",

1 when "00001",

0 when others;

process(CK,start,reset)

constant freq 	: integer := 400;

variable curState : UNSIGNED(1 downto 0) := "11";

variable tick 		: integer := 0;

variable zero 		: std_logic := '0';

variable counter 	: integer;

variable hold : std_logic;

begin

if rising_edge(CK) then

if curState = "11" and start = '1' and hold = '0' then

counter := washC;

curState := "00";

elsif curState = "11" and start = '1' and hold = '0' then

curState := curState;

end if;

if reset = '1' then

curState := "11";

zero := '0';

counter := 0;

tick := 0;

end if;

if zero = '1' then

curState := curState + "01";

zero := '0';

tick := 0;

end if;

if curState = "00" and zero = '0' then

if tick < freq then

tick := tick + 1;

else

tick := 0;

counter := counter - 1;

end if;

if counter = 0 then

zero := '1';

counter := spinC;

end if;

end if;

if curState = "01" and zero = '0' then

if tick < freq then

tick := tick + 1;

else

tick := 0;

counter := counter - 1;

end if;

if counter = 0 then

zero := '1';

counter := rinseC;

end if;

end if;

if curState = "10" and zero = '0' then

if tick < freq then

tick := tick + 1;

else

tick := 0;

counter := counter - 1;

end if;

if counter = 0 then

curState := "11";

end if;

end if;

hold := start;

end if;

tState <= std_logic_vector(curState);

time_sn <= std_logic_vector(to_unsigned(counter, time_sn'length));

end process;

with tState select modes <=

"1000" when "11",

"0100" when "00",

"0010" when "01",

"0001" when "10",

"1111" when others;

state <= modes & "0000" & tState;

end Behavioral;

# UCF

NET "leds[9]" LOC = P77;

NET "leds[8]" LOC = P83;

NET "leds[7]" LOC = P84;

NET "leds[6]" LOC = P86;

NET "leds[5]" LOC = P89;

NET "leds[4]" LOC = P93;

NET "leds[3]" LOC = P3;

NET "leds[2]" LOC = P6;

NET "leds[1]" LOC = P13;

NET "leds[0]" LOC = P16;

NET "Clock" LOC = P40;

NET "reset" LOC = P35;

NET "start" LOC = P15;

NET "SevsegDriver[7]" LOC = P61;

NET "SevsegDriver[6]" LOC = P60;

NET "SevsegDriver[5]" LOC = P57;

NET "SevsegDriver[4]" LOC = P59;

NET "SevsegDriver[3]" LOC = P56;

NET "SevsegDriver[2]" LOC = P52;

NET "SevsegDriver[1]" LOC = P49;

NET "SevsegDriver[0]" LOC = P50;

NET "SevsegData[6]" LOC = P71;

NET "SevsegData[5]" LOC = P62;

NET "SevsegData[4]" LOC = P65;

NET "SevsegData[3]" LOC = P72;

NET "SevsegData[2]" LOC = P73;

NET "SevsegData[1]" LOC = P98;

NET "SevsegData[0]" LOC = P64;

NET "userMode[4]" LOC = P78;

NET "userMode[3]" LOC = P82;

NET "userMode[2]" LOC = P85;

NET "userMode[1]" LOC = P88;

NET "userMode[0]" LOC = P90;

# RLT Schematics

,